|  | Use show vlan brief on old switches to see VLAN IDs, names, and active ports. |
| --- | --- |
| S1 |  |
| S2 |  |
| S3 |  |

|  | Use show interfaces trunk or show running-config to see trunk ports. |
| --- | --- |
| S1 |  |
| S2 |  |
| S3 |  |

|  | Use show etherchannel summary on old switches |
| --- | --- |
| S1 |  |
| S2 |  |
| S3 |  |

| S1 |  |
| --- | --- |

| R1 |  |
| --- | --- |
| R2 |  |

Configure vlan name all switch

conf t
vlan 5
name Staff
vlan 15
name IT
vlan 25
name Anything
exit

ACCESS PORTS (example — adjust per your table)

S2

interface fa0/5
 switchport mode access
 switchport access vlan 5

S3

interface fa0/15
 switchport mode access
 switchport access vlan 15

TRUNK LINKS

S1

interface range fa0/1 - 2
 switchport mode trunk
 switchport trunk allowed vlan 1,5,15,25

S2

interface range fa0/1 - 2
 switchport mode trunk
 switchport trunk allowed vlan 1,5,15,25

S3

interface range fa0/11 - 12
 switchport mode trunk
 switchport trunk allowed vlan 1,5,15,25