1. Passwords

- The exam explicitly says “no need to configure passwords”.

- Also, you only have privilege level 1 on the old devices, which means you can run show commands but cannot change passwords or enter privileged exec mode.

- ✅ So you do not need any passwords for this task.

2. What you need to do

You’re essentially replicating the old network configuration onto the new network. That includes:

A. Switch Configuration

- VLANs

  - Use show vlan brief on old switches to see VLAN IDs, names, and active ports.

  - Configure same VLANs on New_S1, New_S2, New_S3.

- Management VLAN IP

  - Use show running-config on old switches to see the management VLAN interface IP.

  - Assign the same IP on the new switches.

- Trunk Links

  - Use show interfaces trunk or show running-config to see trunk ports.

  - Configure same trunk ports on new switches.

- STP

  - Use show spanning-tree to see STP mode (PVST+, RSTP), root bridge, priorities.

  - Apply same STP settings on new switches.

- EtherChannels

  - Use show etherchannel summary on old switches.

  - Configure the same EtherChannel groups on new switches.

B. Router Configuration

- IP Addressing & Inter-VLAN Routing

  - Use show running-config on old routers to see VLAN interfaces and IP addresses.

  - Configure same IPs and subnets on New_R1 and New_R2.

- HSRP

  - Use show running-config to see HSRP groups, virtual IPs, priorities.

  - Configure same HSRP settings on the new routers.

C. PC Configuration

- Static IPs (given in the scenario):

  - New_PC1 → 5.15.25.50

  - New_PC2 → 15.115.215.100

- Configure subnet masks and gateways consistent with the VLANs.

3. Verification

After configuring, you need to verify full connectivity:

- Ping from each device to every other device.

- Example commands:

  - On PC: ping 5.15.25.1 (router or switch VLAN interface)

  - On router: ping 15.115.215.100 (PC or other VLAN)

✅ Key Notes

- No passwords are required; focus on observing the old network with show commands.

- The critical part is careful observation and replication:

  - VLAN IDs

  - IP addresses and gateways

  - Trunk links and EtherChannels

  - STP and HSRP

STEP 1: Collect Info from OLD Network (IMPORTANT)

Run these on R1, R2, S1, S2, S3:

🔹 Switches (Old S1, S2, S3)

show vlan brief
show interfaces trunk
show etherchannel summary
show spanning-tree
show running-config
show ip interface brief

👉 Write down:

- VLAN IDs + names

- Which ports belong to which VLAN

- Trunk ports

- EtherChannel group numbers

- STP root bridge + priority

- Management VLAN IP

🔹 Routers (Old R1, R2)

show running-config
show ip interface brief

👉 Focus on:

- Subinterfaces (e.g. G0/0.10, .20)

- IP addresses

- HSRP config (standby IP, priority, preempt)

✅ STEP 2: Configure NEW SWITCHES

🔹 1. Create VLANs (on all switches)

conf t
vlan 10
name VLAN10
vlan 20
name VLAN20
exit

(Use exact VLAN IDs/names from OLD network)

🔹 2. Assign Access Ports

Example:

interface fa0/1
switchport mode access
switchport access vlan 10

🔹 3. Configure Trunks

interface g0/1
switchport mode trunk
switchport trunk allowed vlan 10,20

🔹 4. Configure EtherChannel

(Use SAME group number as old network)

interface range g0/1 - 2
channel-group 1 mode active
exit

interface port-channel 1
switchport mode trunk

Check:

show etherchannel summary

🔹 5. Configure STP

Example (if S1 is root):

spanning-tree vlan 10 priority 4096

(Use values from OLD network)

🔹 6. Management VLAN IP

interface vlan 10
ip address 192.168.1.2 255.255.255.0
no shutdown

ip default-gateway 192.168.1.1

✅ STEP 3: Configure NEW ROUTERS

🔹 1. Subinterfaces (Router-on-a-stick)

conf t
interface g0/0.10
encapsulation dot1Q 10
ip address 5.15.25.1 255.255.255.0

interface g0/0.20
encapsulation dot1Q 20
ip address 15.115.215.1 255.255.255.0

(Use EXACT values from OLD network)

🔹 2. HSRP Configuration

On New_R1:

interface g0/0.10
standby 1 ip 5.15.25.254
standby 1 priority 110
standby 1 preempt

On New_R2:

interface g0/0.10
standby 1 ip 5.15.25.254
standby 1 priority 100
standby 1 preempt

(Repeat for other VLANs)

✅ STEP 4: Configure PCs

🔹 New_PC1

- IP: 5.15.25.50

- Mask: 255.255.255.0

- Gateway: 5.15.25.254

🔹 New_PC2

- IP: 15.115.215.100

- Mask: 255.255.255.0

- Gateway: 15.115.215.254

✅ STEP 5: VERIFY (VERY IMPORTANT)

🔹 Check Switch

show vlan brief
show interfaces trunk
show etherchannel summary
show spanning-tree

🔹 Check Router

show standby brief
show ip route

🔹 Ping Tests

From PC1:

ping 5.15.25.254
ping 15.115.215.100

From PC2:

ping 15.115.215.254
ping 5.15.25.50

🚨 COMMON MISTAKES (EXAM KILLERS)

- ❌ Wrong VLAN ID

- ❌ Forgot no shutdown

- ❌ EtherChannel mismatch (group or mode)

- ❌ Trunk not allowing VLANs

- ❌ Wrong HSRP virtual IP

- ❌ Wrong default gateway on PC

🎯 PRO TIP (IMPORTANT)

Don’t guess anything. Always:
👉 Check OLD → Copy EXACT → Apply to NEW